Module -7:

Network fundamental

1 Which of the following messages in the DHCP process are broadcasted? (Choose two)

A. Request

B. Offer

C. Discover

D. Acknowledge

ANS: REQUEST (A) AND DISCOVER (C)

2- Which command would you use to ensure that an ACL does not block web-based TCP traffic?

A. permit any

B. permit tcp any any eq 80

C. permit tcp any eq 80

D. permit any any eq tcp

ANS: (B) permit tcp any any eq 80

3-Explain Network Topologies

Network Topologies

A network topology refers to the physical or logical arrangement of devices (nodes) and connections in a computer network. It defines how computers, switches, routers, and cables are connected.

Below are the main types of network topologies:

1. Bus Topology

- All devices are connected to a single central cable called a bus.

- Data travels in both directions along the bus.

2. Star Topology

- All devices are connected to a central device like a hub or switch.

- Each device has a dedicated connection.

3. Ring Topology

- Each device is connected to two other devices, forming a circular path.

- Data travels in one direction (or both in dual ring).

4. Mesh Topology

- Devices are interconnected with multiple paths.

- Can be full mesh or partial mesh.

5. Tree Topology

- Combination of star and bus topologies.

- Devices are arranged in a hierarchical structure.

4-Explain TCP/IP Networking Model.

ANS: TCP/IP Networking Model

The TCP/IP (Transmission Control Protocol / Internet Protocol) model is a 4-layer networking model used for communication over the Internet and most modern networks. It explains how data is sent, received, and processed across networks.

Layers of TCP/IP Model

1. Application Layer

- Topmost layer of the TCP/IP model

- Provides services directly to the user or application

- Combines the functions of Application, Presentation, and Session layers of the OSI model

2. Transport Layer

- Responsible for end-to-end communication

- Ensures data delivery between sender and receiver

3. Internet Layer

- Handles logical addressing and routing of data

- Determines the best path for data to travel

4. Network Access Layer

- Lowest layer of the TCP/IP model

- Responsible for physical transmission of data

5-Explain LAN and WAN Network.

ANS: LAN and WAN Networks

Networks are classified based on their geographical coverage. The two most common types are LAN (Local Area Network) and WAN (Wide Area Network).

1. LAN (Local Area Network)

A LAN is a network that connects computers and devices within a small geographical area, such as a home, office, school, or building.

Advantages:

- Fast communication

- Easy to manage

- High security

- Low maintenance cost

2. WAN (Wide Area Network)

A WAN connects multiple LANs over a large geographical area, such as cities, countries, or continents.

Advantages:

- Long-distance communication

- Resource sharing across locations

6-Explain Operation of Switch.

ANS: Operation of a Switch

A network switch is a Layer-2 (Data Link Layer) device used in a LAN to connect multiple devices and forward data efficiently using MAC addresses.

How a Switch Works (Step-by-Step)

1. Frame Reception

- The switch receives a data frame from a connected device on a port.

- The frame contains:

  - Source MAC address

  - Destination MAC address

2. MAC Address Learning

- The switch reads the source MAC address.

- It stores this MAC address along with the incoming port number in its MAC address table (CAM table)

- 3. Frame Forwarding Decision

- a) Known Destination MAC

- If the destination MAC address exists in the MAC table:

- The switch forwards the frame only to the specific port connected to that device.

7-Describe the purpose and functions of various network devices.

ANS: Purpose and Functions of Network Devices

Network devices are hardware components used to connect, manage, and control communication between computers and networks. Each device has a specific purpose and function.

1. Hub

Purpose:

- Connect multiple devices in a LAN

Functions:

- Broadcasts data to all connected devices

- Operates at Physical Layer (Layer 1)

2. Switch

Purpose:

- Efficiently connect devices within a LAN

Functions:

- Forwards data using MAC addresses

- Reduces network traffic

- Operates at Data Link Layer (Layer 2)

3. Router

Purpose:

- Connect different networks together

Functions:

- Routes data using IP addresses

- Chooses best path for data

- Operates at Network Layer (Layer 3)

4. Bridge

Purpose:

- Connect two LAN segments

Functions:

- Filters traffic using MAC addresses

- Reduces collisions

5. Repeater

Purpose:

- Extend network distance

Functions:

- Regenerates weak signals

- Operates at Physical Layer

6. Modem

Purpose:

- Connect LAN to Internet

Functions:

- Converts digital signals to analog and vice versa

- Used for ISP connectivity

7. Access Point (AP)

Purpose:

- Provide wireless connectivity

Functions:

- Connects wireless devices to wired LAN

- Used in Wi-Fi networks

8. Firewall

Purpose:

- Protect network from unauthorized access

Functions:

- Filters incoming and outgoing traffic

- Applies security rules

9. Gateway

Purpose:

- Connect different network architectures

Functions:

- Protocol conversion

- Data translation

8.Make list of the appropriate media, cables, ports, and connectors toconnect switches to other

ANS: 1. Switch to Switch

| Medi a | Cable Type | Ports Used | Connectors |
| --- | --- | --- | --- |
| Copper | Ethernet (Straight / Crossover*) | FastEthernet / GigabitEthernet | RJ-45 |
| Fiber | Fiber Optic Cable | SFP / GBIC | LC / SC |

| Media2. Switch to Router | Cable Type | Ports Used | Connectors |
| --- | --- | --- | --- |
| Copper | Ethernet Straight-Through | Ethernet / GigabitEthernet | RJ-45 |
| Fiber | Fiber Optic | SFP Ports | LC / SC |

3. Switch to Computer / Server

| Media | Cable Type | Ports Used | Connectors |
| --- | --- | --- | --- |
| Copper | Ethernet Straight-Through | Ethernet NIC ↔ Switch Port | RJ-45 |
| Fiber | Fiber Optic (Enterprise) | SFP | LC / SC |

4. Switch to Access Point (AP)

| Media | Cable Type | Ports Used | Connectors |
| --- | --- | --- | --- |
| Copper | Ethernet Straight-Through (PoE) | Ethernet / PoE Port | RJ-45 |

5. Switch to Hub

| Media | Cable Type | Ports Used | Connectors |
| --- | --- | --- | --- |
| Copper | Crossover Cable | Ethernet Ports | RJ-45 |

6. Switch to Firewall

| Media | Cable Type | Ports Used | Connectors |
| --- | --- | --- | --- |
| Copper | Ethernet Straight-Through | Ethernet / GigabitEthernet | RJ-45 |
| Fiber | Fiber Optic | SFP | LC / SC |

7. Switch to ISP / Core Network

| Media | Cable Type | Ports Used | Connectors |
| --- | --- | --- | --- |
| Fiber | Single-mode / Multi-mode Fiber | SFP / QSFP | LC / SC |
| Copper | Ethernet | Gigabit / 10G Port | RJ-45 |

9-Define Network devices and hosts

ANS: 1. Network Devices

Network devices are hardware components that help in connecting different devices, controlling data flow, and enabling communication within a network or between multiple networks. These devices mainly work at different layers of the OSI model.

Types of Network Devices

1. Hub

- A hub is a basic networking device.

- It works at the Physical Layer (Layer 1).

- It broadcasts data to all connected devices without checking the destination.

- Causes network congestion and is rarely used today.

2. Switch

- A switch connects multiple devices within a LAN.

- Works at the Data Link Layer (Layer 2).

- Uses MAC addresses to forward data only to the intended device.

- Faster and more secure than a hub.

3. Router

- A router connects two or more different networks.

- Works at the Network Layer (Layer 3).

- Uses IP addresses to route data.

- Commonly used to connect a LAN to the Internet.

- Supports routing protocols like RIP, OSPF, and EIGRP.

4. Bridge

- A bridge divides a network into segments.

- Works at Layer 2.

- Reduces traffic by filtering data based on MAC addresses.

5. Modem

- Converts digital signals into analog and vice versa.

- Used to connect a network to an ISP.

- Examples: DSL modem, Cable modem, Fiber modem.

6. Access Point (AP)

- Allows wireless devices to connect to a wired network.

- Used in Wi-Fi networks.

- Works at Layer 2.

7. Firewall

- Provides network security.

- Filters incoming and outgoing traffic based on rules.

- Prevents unauthorized access.