Module 8:

Network access Basic routing and Advance routing concept , switching concept-

MCQ

- -R1, R2, R3, and R4 have their Fast Ethernet 0/0 interfaces attached to the same VLAN. A network engineer has typed a configuration for each router by using a word processor. He will later copy and paste the configuration into the routers. Examine the following exhibit, which lists configuration for the four routers, as typed by the network engineer. Assuming that all four routers can ping each other’s LAN IP addresses after the configuration has been applied, choose the routers that will be able to form a neighbor relationship with the other routers on the LAN. (You can assume that, if not shown in the exhibit, all other related parameters are still set to their defaults.) (Choose two)

A. R1

B. R2

C. R3

D. R4

E. None of the routers will exchange routing information

ANS- R1 AND R2

2-enable secret hashed using the algorithm.

A. MD5

B. AH

C. PSK

D. ESP

E. WPA2

ANS-   A. MD5

3- An engineer connects to Router R1 and issues a show ip ospf neighbor command. The status of neighbor 2.2.2.2 lists FULL/BDR. What does the BDR mean?

A. R1 is an Area Border Router.

B. R1 is a backup designated router.

C. Router 2.2.2.2 is an Area Border Router.

D. Router 2.2.2.2 is a backup designated router.

ANS -   D. Router 2.2.2.2 is a backup designated router.

4- Which command is used to view the neighbor discovery table on a PC?

A. show ipv6 neighbor

B. show ipv6 neighbors

C. netsh interface ipv6 show neighbor

D. netsh interface ipv6 show neighbors

ANS-   Netsh interface ipv6 show neighbors

6- What type of variable is being shown? Routers = [R1,R2,R3] A. List

B. Dictionary

C. Simple

D. Unsigned integers

ANS-    List

7- Identify the fields in an IPv4 header. (Choose three)

A. Host component

B. Time to Live

C. Source address

D. Destination address

ANS-  Time to Live ,  Source address , Destination address

QUESTION AND ANSWER

- Explain Switch.

ANS-   A Switch is a network device used to connect multiple devices (PCs, printers, servers, IP phones, etc.) within a Local Area Network (LAN) and allow them to communicate efficiently.

It works mainly at the Data Link Layer (Layer 2) of the OSI model. Some advanced switches (Layer 3 switches) can also work at the Network Layer.

Types of Switches

- Unmanaged Switch

  - Plug and play

  - No configuration needed

- Managed Switch

  - Can configure VLAN, security, speed, etc.

  - Used in companies

- Layer 2 Switch

  - Works using MAC addresses

- Layer 3 Switch

  - Can perform routing (uses IP addresses)

### Example

If PC1 sends data to PC3:

  - Switch checks MAC table

  - Sends data only to PC3’s port

  - Other PCs do not receive the data

- Explain Switch Boot Sequence

ANS- The Switch Boot Sequence is the step-by-step process that a switch follows when it is powered ON.

When you turn on a Cisco switch, it loads the Operating System (IOS) and configuration files before it becomes ready to use.

Steps in Switch Boot Process

- Power On Self Test (POST)

- When the switch is powered ON, it runs POST.

- It checks:

  - CPU

  - RAM

  - Flash memory

  - Interfaces (ports)

- If hardware fails → switch shows error (LED indication).

- Load Boot Loader

- The Boot Loader program is stored in ROM.

- It initializes:

  - CPU

  - Flash file system

  - Low-level hardware

- Its main job is to load the IOS.

- Initialize Flash File System

- Boot loader checks Flash memory.

- Looks for the IOS image file.

Load Cisco IOS into RAM

- The IOS image is copied from Flash → RAM.

- IOS runs from RAM.

- If IOS not found → switch enters ROMMON mode.

- Load Startup Configuration

- Switch checks NVRAM for startup-config.

- If found → loads configuration into RAM (running-config).

- If not found → enters Setup mode.

- Switch is Ready

- Loads VLAN database (from flash).

- Ports become active.

- Switch is ready for operation.

3. Explain Three Methods to access Switch Command Line Interface

ANS- A switch CLI (Command Line Interface) can be accessed in three main ways:

- Console Port Access

- Connect PC to switch using console cable (rollover cable).

- Use software like PuTTY / Tera Term / HyperTerminal.

- Direct physical connection.

- Does not require IP address.

- Used for:

- Initial configuration

- Troubleshooting

- Telnet

- Remote access method over network.

- Requires:

  - Switch IP address configured

  - Telnet password set

- Less secure (password sent in plain text).

- SSH (Secure Shell)

- Secure remote access method.

- Encrypts data.

- Requires:

  - IP address

  - Domain name

  - RSA key generation

  - Username & password

4.Explain and Configuring the Cisco Internet Operating System.

ANS- 🔹 What is Cisco IOS?

Cisco IOS (Internet Operating System) is the operating system used in Cisco routers and switches.
It controls:

- Network traffic forwarding

- Routing and switching functions

- Security features

- Device management

IOS provides a Command Line Interface (CLI) to configure and manage the device.

Basic IOS Configuration (Switch Example)

- Enter Privileged Mode

enable

- Enter Global Configuration Mode

configure terminal

- Set Hostname

hostname Switch1

- Set Enable Password

enable secret cisco

- Configure Console Password

line console 0

password 1234

login

exit

- Configure VTY (Telnet) Password

line vty 0 4

password 1234

login

exit

- Configure IP Address (Management VLAN)

interface vlan 1

ip address 192.168.1.10 255.255.255.0

no shutdown

exit

- Set Default Gateway

ip default-gateway 192.168.1.1

- Explain Switch Port

ANS- A Switch Port is a physical interface on a network switch used to connect devices like PCs, servers, printers, or other switches using Ethernet cables.

Each port on a switch acts as a separate communication channel.

Functions of a Switch Port

- Connects end devices to the network

- Sends and receives data frames

- Identifies devices using MAC addresses

- Can be configured for VLAN, speed, and security

Types of Switch Ports

- Access Port

- Connects to end devices (PC, printer, IP phone)

- Carries traffic for only one VLAN

- Most commonly used port type

- Trunk Port

- Connects to another switch or router

- Carries traffic for multiple VLANs

- Uses VLAN tagging (802.1Q)

Important Port Settings

You can configure:

- Speed (10/100/1000 Mbps)

- Duplex (half/full)

- Port security

- VLAN assignment

- Enable/Disable port (shutdown / no shutdown)