Module: 13- Networking with Windows Server

25. Discuss the role of Windows Firewall in Windows Server and how to configure it.

Ans. Windows Firewall is a built-in security feature in Windows Server that helps protect your server from unauthorized access and cyber threats.

Main Roles:

- Traffic Control: It monitors incoming and outgoing network traffic and blocks or allows it based on rules.

- Security Layer: Acts as the first line of defense against hackers, malware, and unauthorized users.

- Port Filtering: Blocks unused or risky ports (like preventing unwanted services from being accessed).

- Profile-based Protection: Applies different rules depending on network type:

  - Domain (connected to company network)

  - Private (trusted internal network)

  - Public (untrusted network like Wi-Fi)

26. What is Network Address Translation (NAT) in Windows Server, and how do you configure it?

Ans. Network Address Translation (NAT) in Windows Server is a feature that allows multiple devices on a private network (like 192.168.x.x) to access the internet using a single public IP address.

In short- NAT hides your internal network and lets all devices share one internet connection.

- How to Configure NAT in Windows Server

To configure Network Address Translation (NAT) in Windows Server, you typically use the Routing and Remote Access Service (RRAS) feature. First, install the Remote Access role with routing enabled through Server Manager. After installation, open the RRAS console, enable and configure the service by selecting the NAT option, and then choose the network interfaces—one connected to the internet (external) and one connected to the internal network. The external interface is configured to perform NAT so that it translates private IP addresses into a public IP. Once the service is started, all devices on the internal network can access external networks through the server using NAT, without needing individual public IP addresses.

27. Explain the concept of Dynamic Host Configuration Protocol (DHCP) and how to configure it in Windows Server 2016.

Ans. Concept of DHCP

- dynamic Host Configuration Protocol (DHCP) is a network service that automatically assigns IP addresses and other network settings (like subnet mask, default gateway, and DNS) to devices on a network. In Windows Server 2016, DHCP removes the need to manually configure IP addresses on each client machine. When a device connects to the network, it sends a request, and the DHCP server responds by leasing an available IP address from a defined range (scope). This process is commonly known as DORA

- DHCP Configuration in Windows Server 2016

To configure DHCP in Windows Server 2016, you first install the DHCP Server role using Server Manager. After installation, you open the DHCP management console and create a scope, which defines the range of IP addresses to be distributed to clients. While creating the scope, you specify important settings such as subnet mask, IP range, exclusions, lease duration, default gateway, and DNS server details. Once the scope is activated and the DHCP server is authorized in the domain, it starts automatically assigning IP addresses to client systems on the network.

28.Describe the process of configuring DNS (Domain Name System) in Windows Server.

Ans. Step 1: Install DNS Role

- Open Server Manager

- Click Add Roles and Features

- Select DNS Server role

- Click Next → Install

Step 2: Open DNS Manager

- Go to Tools

- Click DNS
➡️ DNS Manager window will open

Step 3: Create Forward Lookup Zone

- Right-click Forward Lookup Zones

- Click New Zone

- Choose:

  - Primary Zone

- Enter Zone Name (e.g., example.com)

- Finish the wizard

Step 4: Add DNS Record (A Record)

- Open your created zone

- Right-click → New Host (A or AAAA)

- Enter:

  - Name (e.g., www)

  - IP Address (e.g., 192.168.1.10)

- Click Add Host

Step 5: (Optional) Create Reverse Lookup Zone

- Right-click Reverse Lookup Zones

- Click New Zone

- Select Primary Zone

- Enter Network ID (e.g., 192.168.1)

- Finish setup

Step 6: Test DNS

- Open Command Prompt on client PC:

- nslookup www.example.com

- If it shows IP → DNS working

29. What is Server Manager, and how do you use it to manage servers in Windows Server?

Ans. Server Manager is a built-in management tool in Windows Server that allows administrators to install roles and features, monitor system status, and manage multiple servers from one place. It provides a centralized dashboard to handle tasks like configuring services (DNS, DHCP, AD), checking performance, and managing remote servers.

- How to Use Server Manager

- 1. Open Server Manager

- 2. Add Roles and Features

- 3. Manage Installed Roles

- 4. Add and Manage Multiple Servers

- 5. Monitor Server Health

- 6. Use Tools Menu

30. Discuss the role of Remote Desktop Services (RDS) in Windows Server 2016 or 2019 and how to configure it.

Ans. Role of Remote Desktop Services (RDS)

Remote Desktop Services (RDS) in Windows Server 2016 / Windows Server 2019 allows users to access a server or desktop remotely over a network. It enables multiple users to log in at the same time and use applications or desktops hosted on the server.

Key Roles:

- Remote Access: Users can connect from anywhere using Remote Desktop

- Centralized Applications: Run apps on the server instead of installing on every PC

- Multi-user Environment: Many users can work on the same server simultaneously

- Cost Effective: Reduces need for powerful client machines

How to Configure RDS

Step 1: Install RDS Role

- Open Server Manager

- Click Manage → Add Roles and Features

- Select Remote Desktop Services Installation

- Choose Quick Start (easy setup)

- Select:

  - Session-based desktop deployment

- Choose your server → Click Install

Configure Deployment

- Server will automatically install:

  - RD Session Host

  - RD Web Access

  - RD Connection Broker

Wait for installation to complete and server may restart

Step 3: Allow Users

- Go to Server Manager → Remote Desktop Services

- Click Collections

- Select your collection

- Add users/groups who can access RDS

Step 4: Enable Remote Access

- Ensure Remote Desktop is enabled:

  - Go to System Properties → Remote

  - Select Allow remote connections

Step 5: Test Connection

- On client PC:

Mstsc  ping

enter server IP → Login