Assignment: module -5 Network Fundamentals and Building Networks

Section 1: Multiple Choice

1. What is the primary function of a router in a computer network?

a) Assigning IP addresses to devices

b) Providing wireless connectivity to devices

c) Forwarding data packets between networks

d) Managing user authentication and access control

ANS:  Forwarding data packets between networks

2. What is the purpose of DHCP (Dynamic Host Configuration Protocol) in a computer network?

a) Assigning static IP addresses to devices

b) Resolving domain names to IP addresses

c) Managing network traffic and congestion

d) Dynamically assigning IP addresses to devices

ANS: Dynamically assigning IP addresses to devices

3. Which network device operates at Layer 2 (Data Link Layer) of the OSI model and forwards data packets based on MAC addresses?

a) Router

b) Switch

c) Hub

d) Repeater

ANS: Switch

Section 2: True or False

- A VLAN (Virtual Local Area Network) allows network administrators to logically segment a single physical network into multiple virtual networks, each with its own broadcast domain.

ANS: True

- TCP (Transmission Control Protocol) is a connectionless protocol that provides reliable, ordered, and error-checked delivery of data packets over a network

ANS: False

- A firewall is a hardware or software-based security system that monitors and controls incoming and outgoing network traffic based on predetermined security rules.

ANS: True

8. Describe the steps involved in setting up a wireless network for a small office or home office (SOHO) environment.

ANS: Setting up a wireless network for a Small Office/Home Office (SOHO) environment involves several key steps to ensure reliable connectivity, security, and scalability.

- Assess Network Requirements

Before purchasing equipment or configuring devices:

- Determine user needs: How many users and devices will connect

- Estimate bandwidth requirements: Based on activities

- Consider coverage area: Size of the office, number of rooms, and any physical obstructions

- Choose Appropriate Networking Equipment

Select hardware suitable for your needs:

- Modem: Connects to your Internet Service Provider (ISP).

- Wireless Router or Access Point   Distributes wireless (Wi-Fi) signal.

- Switch   For additional wired connections.

- Wireless Network Interface Cards   For computers without built-in Wi-Fi.

- Physically Set Up the Network

- Configure the Router or Access Point

Access the router’s web-based setup interface (usually via a browser):

- Enter the router’s IP address (e.g., 192.168.0.1 or 192.168.1.1).

- Configure and Connect Client Devices

- On each device (laptop, phone, etc.), open Wi-Fi settings.

- Select your SSID and enter the Wi-Fi password.

- Test connectivity by browsing the Internet or accessing shared resourse

- Secure the Network

- Optimize Network Performance

- Test and Monitor the Network

- Check signal strength in different areas.

- Test Internet speed (e.g., using Speedtest.net).

- Verify security by ensuring only authorized devices can connect.

- Monitor traffic and logs periodically for unusual activity.

Diagram of  a small office or home office (SOHO) environment:

Section 4: Practical

- Demonstrate how to configure a router for Internet access using DHCP (Dynamic Host Configuration Protocol).

ANS: Here’s a step-by-step demonstration of how to configure a router for Internet access using DHCP

Step 1: Physical Connections

- Turn off the modem and router.

- Connect the modem to the router’s WAN (Internet) port using an Ethernet cable.

- Connect your computer to one of the router’s LAN ports (e.g., LAN1).

- Power on the modem, wait 1–2 minutes, then power on the router.

Step 2: Access the Router’s Configuration Interface

- Open a web browser on your computer.

- In the address bar, type the router’s default IP address (usually one of these):

  - 192.168.0.1

  - 192.168.1.1

  - 10.0.0.1

- Press Enter.

- Log in with the default username and password (often printed on a label on the router).

Example: Username: admin, Password: admin or password

Step 3: Configure the WAN (Internet) Settings

- In the router’s admin menu, look for “Internet Setup,” “WAN Setup,” or “Network Settings.”

- Under Internet Connection Type, select DHCP (sometimes called “Dynamic IP” or “Automatic Configuration – DHCP”).

- Click Save or Apply.

Step 4: Verify the Internet Connection

- Go to the Status or Network Map page of the router.

- Check the WAN IP Address section — it should now display an IP address assigned by the ISP.

  - Example: WAN IP: 203.0.113.24

  - If it shows 0.0.0.0, restart the modem and router.

- Open a web browser and try visiting a website

  - If the page loads, Internet access is working correctly.

Step 5: Configure the LAN and Wireless (Wi-Fi) Settings

- Go to Wireless Settings:

  - Set SSID (network name): e.g., Office_WiFi.

  - Choose WPA2 or WPA3 security.

  - Set a strong Wi-Fi password.

- Go to LAN Settings (optional):

  - Ensure DHCP Server is enabled on the router for local devices.

  - This will automatically assign IPs (like 192.168.1.x) to computers and phones.

- Save and reboot the router if required.

Step 6: Test and Troubleshoot

- Check LAN IP: Your computer should have an IP like 192.168.1.2.

- Ping test:

  - Open Command Prompt → type ping 8.8.8.8 → press Enter.

  - Successful replies mean Internet connectivity is working.

- If no connection:

  - Restart modem and router.

  - Verify DHCP is enabled on the WAN interface.

  - Check cables and ISP service.

Section 5:

10.Discuss the importance of network documentation in the context of building and managing networks.

ANS: Network documentation is a critical component in building, managing, and maintaining any computer network — whether small (like a SOHO) or enterprise-level. It involves systematically recording all details about the design, configuration, devices, connections, policies, and procedures that make up the network.

1. Facilitates Network Planning and Design

- Proper documentation helps network administrators plan the layout and topology of the network before implementation.

- It allows for better capacity planning, identifying what equipment, cables, and IP addressing schemes are required.

2. Simplifies Network Implementation

- During the installation or upgrade process, well-prepared documentation acts as a blueprint.

- Reduces confusion and ensures that the actual setup matches the intended design.

3. Aids in Troubleshooting and Problem Resolution

- When a network issue arises, accurate documentation helps identify where the problem might be.

- For example, knowing which switch port a device connects to or the configured IP address range speeds up diagnosis.

4. Enhances Network Security

- Documentation includes records of user access controls, firewall rules, and security configurations.

- Makes it easier to review and audit security settings regularly.

5. Supports Network Maintenance and Upgrades

- When making changes (like adding new devices or upgrading firmware), documentation ensures changes are made consistently and safely.

- Minimizes the risk of disrupting other services

6. Improves Team Collaboration and Knowledge Transfer

- Network documentation ensures that information is not dependent on one individual.

- Encourages standardization and consistency across the organization.

7. Assists in Asset and Inventory Management

- Documentation tracks all network devices (routers, switches, access points, cables, etc.) along with serial numbers, IPs, and firmware versions.

8. Ensures Compliance and Audit Readiness

- Many organizations must comply with standards (e.g., ISO, PCI-DSS, HIPAA).

- Avoids penalties during audits

9. Enables Disaster Recovery and Business Continuity

- In the event of a failure, natural disaster, or cyberattack, documented network configurations and topology maps are vital for restoration.

10. Types of Network Documentation

Effective documentation usually includes:

- Network topology diagrams (physical and logical)

- IP addressing and subnetting plan

- Device inventory list

- Configuration files and backup procedures

- Cabling layouts and labeling schemes